Supercharger Formulae:

| Calculating Pulley Ratio: |  |
| --- | --- |
| Pulley Ratio (PR) | = C(14.7+B+I)/14.7xV |
|  | = Crank pulley Diameter/Blower pulley Diameter |
|  | = Crank R.P.M./Blower R.P.M. |
| Where | C = ½ Engine Capacity in c.c. |
|  | B = boost in P.S.I. |
|  | V = blower volume in c.c. (bear in mind that Roots blower volume is “displacement” and vane blower volume is “sweep volume” - as described above) |
|  | I = Allowance for valve overlap (i.e. for every 10 degrees of overlap allow 5% of the boost. Eg for 35 degrees….. I = 35 x 0.05 x B) |

| Calculating the Change of Compression Ratio: |  |
| --- | --- |
| RB/RA | = square root of the fraction (14.7 + B)/14.7 |
| Where | RA = unknown compression ratio |
|  | RB = new “effective” compression ratio |
|  | B = boost in P.S.I. |

| Calculation the New Peak R.P.M. : | Note….. The engine “power band” will increase by the same ratio |
| --- | --- |
| New Peak R.P.M. | = Original peak R.P.M. x the cube root of the fraction (14.7 + B)/14.7 |
| Where | B = boost in P.S.I. |